Environmental Impact Assessment (EIA)

Unit – I

Basic Concepts of EIA

Environment: Environment means anything around us or surroundings around us.

Components of Environment:

Consists of Two Components

- Biotic Components or Living Components

Ex: Man, Plants, Animals, Microbes etc.

- Abiotic Components or Non-living Components

Ex: Air, Water, Land, Food etc

Introduction: It is a systematic approach involves in assessment of impacts of development projects on Environmental components.

Ex: Dams, Road and Railway Lines, Industries, Mining, highways, bridges etc.

Development Projects:

- A project or an activity which provides lot of economy or generates huge income to the govt people, called as ‘Development Project or activity’.

Ex: Dams, Road and Railway Lines, Industries, Mining, Highways, Bridges etc.

Impact

Impact is any change in the physical, chemical, biological, cultural or socio-economic environmental system, could be adverse or beneficial.

Objectives of EIA

- To identify the environmental, social and economic impacts of a project prior to taking a decision on its implementation.

- Mitigation (reducing the severity) of harmful impacts and maximising the beneficial effects. Once the assessment is complete, the EIA findings are communicated to stake holders i,.e, developers, investors, regulators, planners, politicians, affected communities etc. Based on the conclusion of the EIA process, the govt can decide if a project should be given environment clearance or not.

- The developers and investors can shape the project so as to mitigate its harms and maximise its benefits.

CONCEPTS OF EIA

EIA is a procedure to plan some developmental activity with well-defined environmental goals so that damage due to the activity both during developmental stage and production stage have minimum impact on the natural system and the population in the area.

EIA Guidelines:

The National Environmental Policy Act (NEPA) USA in 1969 first provided the guidelines for Environmental Impact Assessment through Council for Environmental Quality (CEQ).

Gazette Notification for EIA;

In India the Gazette Notification of EIA was issued in 1994 in which the Ministry of Environment and Forest (MoEF) provided guidelines for project proponents to have EIA and prepare Environmental Impact Statement (EIS) prior to the clearance of the project.

GOALS OF EIA

- To fulfil the responsibilities towards the coming generations as trustees of the environment.

- To assure safe, healthy, productive as well as culturally pleasing surroundings.

- To provide widest range of beneficial uses of environment without degradation or risk to health.

- To preserve historical, cultural and natural heritage.

- To achieve a balance between population and resource use for a good standard of living.

- To ensure sustainable development with minimal environmental degradation.

Factors affecting EIA/ Land Clearing Activities (LCA).

The major impacts of LCA project on environment are

- Physical Resources

- Ecological Imbalance

- Human Use Values

- Quality of Life Values

1) Physical Resources:  Physical resources are

a) Soil quality

b) Water

c) Plants

d) Animals etc.

- Soil Quality: due to deforestation, construction of dams, roads and railway lines, soil quality would be affected. As

- Soil erosion occurs.

- Soil fertility is lost due to physical stresses involved in cleaning and levelling.

- Loss of rain water infiltration (infiltration is the process by which water on the ground surface enters the soil).

- Micro effects on increasing temperatures (with temperature increase from 0 to 10 degrees, the algal community changes).

2) Ecological Imbalance

-Forest resources (wild life habitat) are lost.

- Hazards may encroach to nearby forests due to agricultural development.

- Hazards from pesticides and other agricultural toxins.

3) Human Use Values

- Impairment(damage) of downstream water quality

- community water supply

beneficial water is lost as run off  (run off is the part of the water cycle that flows over land as surface water instead of being absorbed into ground or evaporating) Sedimentation and flooding hazards.

4) Quality of Life Hazards

-Loss of forest tourism

- Impairment of aesthetic values

- Disruption of local forest population

- Disease hazards (due to insects etc.) to farmer population.

- Sanitation disease hazards due to increases population.

Salient Features of EIA

- EIA identifies the positive and negative impacts of a proposed project on the environment.

- These impacts can be short term or long term.

- EIA a plan which when implemented reduces the negative impacts so that there is a minimal level of degradation.

- The plan also utilises the positive impacts which offset the negative impacts.

- EIA provides a monitoring program to measure the level of plan implementation and the degree of effectiveness.

EIA Procedure:

The entire EIA procedure can be divided into 2 tasks or sub reports.

- The Initial Environmental Examination (IEE)

and

- The Full scale Environmental Impact Assessment (EIA).

- Initial Environmental Examination (IEE)

- Involves project screening to determine which projects require a full scale EIA.

- -IEE ensures project oriented environmental management and also minimises the effort, expense and delay in carrying out such planning.

- Based on the information and judgement given by an expert, IEE assesses the potential environmental effects of a proposed project.

- If the IEE results indicate that a full scale EIA is not required, then any environmental management parameters such as environmental protection measures or a monitoring program can be adapted.

- If on the other hand, full scale EIA is required, IEE gives a mechanism to determine and identify the key issues and also those issues which require only a cursory discussion.

- IEE provides very efficiently provides the environmental management plans with or without the requirement of a full scale EIA.

- Therefore for most industrial developmental projects, IEE is desirable from the economic point of view

Important steps in Full Scale Environmental Impact Assessment

EIA is a multidisciplinary approach which- identifies, assesses and analysis the impacts of developmental projects on natural resources such as air quality , water quality, soil quality, noise, plants, animals etc.

EIA Process Flow Chart:

- Screening: -

- Screening is the first and simplest process in project evaluation. It is done to see whether the project needs an EIA for clearance or not.

Further there are some prohibited areas where generally development projects are not allowed.

Ex:- Coastal Regulation Zone(CRZ)

Dahanu Taluka in Maharashtra

Aravalli Range, Reserve Forests

Sensitive Zones etc.

Type of Projects:

The government rules, categorize projects into two categories, A and B based on the spatial extent of the impacts, effects on human health and the effects on the environment.

1. Category -A projects are looked into by the central govt.

2. Category -B projects go to the state govt.

Category B projects are further sub divided in to category B1 and category B2.

A.B1 Projects require a public hearing for EIA.

B.B2 don’t require.

- Scoping:-

Scoping is another stage before the main EIA process begins. It would study and address all issues of importance and the concerns raised by various groups.

- It involves determination of the extent of EIA required for the project. Depending upon the project, basically two types of EIA may be carried out.

- When EIA report is based on a single season data, it is called “Rapid EIA”.

- When the EIA report is based on detailed seasonal data, it is called “Comprehensive EIA”.

- Public hearing:-

The SPCB conducts a public hearing at the site (or) in its close proximity. It includes obtaining responses in writing or oral from the affected persons and other persons by posting on website within   7days of receiving application.

Public hearing is not required for

- Small scale industrial undertaking located in notified or designated industrial areas/industrial estates.

- Areas marked for industries under the jurisdiction of industrial development authorities.

- Widening and strengthening of high ways.

- Minor projects with lease areas up to 25 hectares

- SEZ s and

- Modernization of existing irrigation projects.

- Baseline Data  Acquisition:

It gives a holistic picture of the overall environmental setting of the project location showing any significant environmental items prior to initiation of the action, any potentially critical environmental changes and information about the site to the decision makers and receivers, who might be unfamiliar with the general location of the project area.

Environmental Parameters Considered for Preparation of Baseline Data Acquisition:

The following environmental parameters are usually considered while preparing the baseline data.

- Site Location and Topography:

a. Regional Demography:-population distribution with in 10 and 50 km radius, land use and water use pattern.

b. Regional  Land Marks:- like historical and cultural heritage in the area. For this archaeological or state register can be checked.

Geology: - Land forms, soil type, land scape etc are studied.

d. Hydrology: - Ground water and surface water resources are quantified. Water quality, pollution sources etc., are studies.

e. Meteorological: - temperature extremes, wind speed and direction, dew point, atmospheric stability, rain fall, storms etc are checked.

f. Ecology; - The flora, fauna, endangered species, successional stage etc., are enlisted.

- Impact Identification:-

It includes the details of project characters and baseline environmental characteristics to ensure the identification of full range of environmental impacts. During identification process, the positive and negative, direct and indirect, significant and insignificant impacts are considered

- Impact prediction:-

Here magnitude of changes going to occur due to the project are predicted by using mathematics models or mass balance models.

- Impact Analysis:-

This stage of EIA identifies and predicts the likely environmental and social impact of the proposed project and evaluates the significance. It is done by considering the costs and benefits of the projects. Long term effects and side effects of the project are also evaluated. Indirect valuation of environmental parameters are also done.

Ex:- loss of rare species, degradation of a lake etc.

- Mitigation:

Once the impacts are predicted and evaluated, mitigation measures are to be suggested to avoid, reduce or rectify the adverse changes due to the project. Review and a draft impact statement at this stage .The step in EIA recommends the actions to reduce and avoid the potential adverse environmental consequences of development activities.

- Reporting:-

This stage presents the results of EIA in the form of a report to the decision making body and other interested parties.

- Review of EIA:-

It examines the adequacy and effectiveness of the EIA report and provides the information necessary for decision making.

- Decision Analysis:-

Public participation is involved by arranging group decision about the project and its evaluation.

- Decision making:-

It decides whether the project is rejected, approved or needs further change.

- Post monitoring:-

This stage comes into play once the project is commissioned. It checks to ensure that the impacts of the project do not exceed the legal standards and implementation of the mitigation measures are in the manner as described in the EIA report.

- Environmental Impact Statement(EIS)

Based on the data obtained and review suggestions, a final EIS prepared as per the format provided by the MoEF (Ministry of Environment and Forests) in our country.

In EIS clearly mentions the objectives of the project, its environmental impacts , impacts that  are unavoidable , mitigate measure to minimize the impacts, alternatives to the proposed action etc.

- Environmental Audit:-

It compares the impacts predicted in EIS before the project was started and actual impacts after implementation of the project.

Types of EIA

EIA is mainly of 2 types

1) Comprehensive EIA

2) Rapid EIA

1)  Comprehensive EIA:-

If EIA report incorporates the data of all 3 seasons of a year, it is comprehensive EIA.

- Rapid EIA:-

If EIA report has only one season data, it is rapid EIA.

- EIA was introduced in India in 1978 w.r.t River Valley Projects. Later EIA legislation was enhanced to include other developmental sections.

- EIA comes under the notification of developmental projects 1994 under the provision of Environmental (Pollution) Act 1986.

- Besides EIA, the Govt of India under Environment Protection Act 1986 issued a no of other notifications, which are related to EIA.

- EIA is now mandatory for more than 30 categories of projects and these projects get Environment Clearance (EC) only after the EIA requirements are fulfilled.

- EC or go ahead signal is granted by the impact assessment agency in the Ministry of Environment and Forests, Govt. of India.

All the projects that require clearance from Central Govt. can be broadly categorised in to the fall.

- Individual projects that need clearance from Central Govt.

- Nuclear Power and related projects.

- River Valley Projects including Hydel Power, Major Irrigation and Flood Control.

- Ports, Harbours, Airports.

- Petroleum Refineries including Crude and product Pipelines.

- Chemical Fertilizers and Pesticides.

- Bulk Drugs and Pharmaceuticals.

- Thermal Power Plants

- Distilleries

- Pulp, Paper and News Print, Dyes.

- Cement

- Electroplating

- Mining Projects.

Environmental Appraisal:-

An Appraisal Committee constituted by the Ministry of Environment and Forests (MoEF) will first scrutinize a project based on the data presented by the project authorities.

Single Window Clearance:-

-     Environmental Clearance + Forestry Clearance

-     When a project requires both EC and approval under the forest conservation Act of 1980, the proposals should be given simultaneously to the concerned divisions of the ministry.

Main Participants of EIA:-

EIA applies to public and private sections.

The Six Main Participants are

- Those who propose the project/ Project Proponent.

- The Environmental Consultant who prepares EIA on behalf of project proponent.

- Pollution Control Board (State or National)

- Public who has the right to express their opinion.

- The Impact Assessment Agency.

- Regional Centre of the Ministry of Environment and Forest (MoEF).

Composition of the Expert Committees for EIA

- The committees will consists of experts in the following disciplines.

- Ecosystem Management

- Air/Water Pollution Control

- Water Resource Management

- Flora/Fauna Conservation and Management

- Land Use Planning

- Social Sciences/Rehabilitation

- Environmental Health

Amendment of EIA Notification:-

EIA notification of 2006 has decentralised (meaning transfer from central to local). The environmental clearance projects by categorising the developmental projects as Category A and Category B.

After 2006 amendment the EIA cycle comprises of 4 stages

-    Screening

- Scoping

- Public hearing

- Appraisal

Benefits of EIA:-

- EIA links environment with development for environmentally safe and sustainable development.

- EIA provides a cost effective method to eliminate or minimise the adverse impacts of the developmental projects.

- EIA enables the decision makers to analyse the effects of developmental activities on the environment well before the project is implemented.

- EIA encourages the mitigation practices.

- EIA makes sure that the developmental plan is environmentally sound and within the limits of the capacity of assimilation and regeneration of the ecosystem.

Environmental Sensitive Areas:-

There are many areas/places environmentally sensitive and are not suitable for development activity.

a) Ecologically sensitive areas such as

- Religious and historic places

- Archaeological monuments

- Hilly areas

- Coastal resorts

- Health resorts

- Biosphere reserves

- Estuaries

- National parks and Sanctuaries

- Lakes, rivers

- Tribal settlements

- Boarder areas

- Airports etc.

B ) Flood plain riverine system

C ) Major settlements with population more than 3 lakhs

d)  Settlements with lesser population

Environmental Impacts

Environmental impacts arising from any development project fall into three categories.

- Direct impacts

- Indirect impacts

- Cumulative impacts

These three groups can be further broken down according to their nature into

- Positive and negative impacts

- Predictable and Unpredictable /random impacts

- Local and Wide spread impacts

- Short and Long term impacts

Environmental Parameters:

The following environmental parameters to be considered for EIA of any developmental activity.

- Crop Productivity

- Air Quality

- Water Quality and aquatic resources

- Nutrient status of water

- Drinking water quality

- Availability of agricultural land

Components or Elements of EIA:

-Air Environment

-Noise Environment

-Water Environment

- Biological Environment

- Land Environment

- Socio Economic and Health Environment

- Risk Assessment

- Environmental Management Plan - EMP

Air Environment

- Screening to determine the impact zone

- Developing a monitoring network.

- The ambient (surroundings) air quality up to 7-10 km of the proposed project site is monitored with respect to the -wind speed, direction, humidity, ambient temperature, environmental lapse rate (it is the rate at which the air temperature changes with height in the atmosphere).

- Estimation of air emissions (fugitive emissions and vehicular traffic emissions) by identifying, quantifying and evaluating within the impact zone and estimating the impacts.

Fugitive emissions are emissions of gases or vapours from pressurised equipment due to leaks and also from other industrial activities.

Noise Environment

- Monitoring the present status of noise levels and predicting the future noise levels resulting from the proposed project.

- Identifying the impacts and recommendation of mitigation measures.

Water Environment

- Studying the existing ground and surface water resources w.r.t quantity and quality.

- Predicting the impacts due to the project ( as a result of pumping for the project activities etc.)

- Quantification of waste water

- Evaluation- assessment of feasibility of water recycling, reuse and delineation of the plan.

- Suggestions on modifications.

Biological Environment

- Survey of flora and fauna

- Assessment of damages to aquatic and terrestrial flora and fauna due to discharge of gaseous emissions and air pollution from the project.

- Delineation of mitigation measures to prevent or reduce the damage.

Land Environment

- Studying the soil characteristics, land topography, landscape and drainage patterns.

- Estimating the impacts

- Identifying the potential utility of treated effluent in land application.

- Delineation of options for minimisation of waste and environmentally compatible disposal.

Socio Economic and Health Environment

- Collecting demographic and related socio economic data

- Collecting epidemiological data eg. Fluorosis, Malaria, Fileria, Malnutrition etc.

- Morbidity rates among population.

- Assessment of impact on historical, cultural and archaeological sites in the project site.

- Assessment of economic benefits.

Risk assessment

- Identification of hazards, inventory analysis, dam break probability, natural hazard probability.

- Maximum credible accident (MCA) analysis accidents resulting in fire, explosion etc.

- Hazard and operability (HAZOP) studies.

- Assessment of risk of the above

- Preparation of onsite and offsite disaster management plan.

Environment Management Plan (EMP):

Delineation of mitigation measures including prevention and control for each environmental component, rehabilitation and resettlement plan.

Environmental Base Map:

- A base map is a context map. Information can be added to a base map by overlaying other information on top of it.

- Base map contains reference information which provides geospatial information based on what the cartographer is trying to communicate.

- The function of the base map is to provide background detail necessary to orient the location of the map.

- The term base map is seen often in GIS (Geographic Information System Mapping) and refers to collection of GIS.

- Base map was prepared by taking developmental plan 1991 as a reference to digitise roads, railway lines .It was further corrected by overlaying the map on the satellite image.

Five steps

- 1- Contributing specialists

- 2-Land use system

- 3- Land degradation

- 4- Sustainable land management

- 5- Expert recommendation